DIY SOLAR HOT WATER SEMINAR

## NOTES FOR YOU TO TAKE HOME

### ALBIE BURGERS, KOTINGA, TAKAKA

albie.fill@gmail.com

www.esypsy.co.nz

ALBIE'S IDEA NO. 1  (Borrowed from Llyn Richards, Awaroa)

Throw 100m of 15mm black polyethylene pipe onto a flat, black butyl roof.

Fill the pipe with water from the garden hose, then let it lie there on the roof, for 20 minutes in the sun.

Fit a shower rose to the other end of the black pipe, turn on the garden hose, and ...

You've got a 1 minute hot shower!  (10 litres of piping hot water (pardon the pun) because 100m of 15mm pipe holds 10 litres of water.)

Slight modification (because the water is usually too hot!)

Connect a cold water line with a tap between the garden hose and the shower head, to let in some cold water so you don't burn yourself...

There are 2 disadvantages:

- There's just enough time to get yourself soaped before you run out of hot water and have to wait another 20 minutes before you can rinse off.

- This only works while the sun is actually shining.  There's no storage of hot water.

There is one advantage:

- It's simple.

ALBIE'S IDEA NO. 2: Same as no. 1, but with a pump and storage tank

Solar panel: 6m x 5m (30 sq.m) and runs at around 25% efficiency (8kW)

Collector piping: 460m of 15mm LDPE under Agphane (greenhouse plastic)

The pump switches ON when the sun shines and heats the panel above 50deg.C

The collector is plumbed into our standard low pressure hot water system as shown.

NOTE:  This system is ILLEGAL, because water heated inside polypipe is non-potable, and is not allowed to be plumbed into the hot water system.  It requires a heat exchanger to separate the non-potable solar heated water from the water which goes to the hot taps.

ALBIE’S IDEA NO. 3:  Using a LARGE storage tank

This system solves the problem in no. 2 above.  The hot water which goes to the taps is completely isolated from the non-potable hot water in the tank, by the use of a copper coil heat exchanger.  It's ok to use the non-potable water to heat the radiator panels, though.

ALBIE’S IDEA NO. 4:  Using a VERY large storage tank for winter heating

This is called “inter-seasonal” heat storage (or “long-term” heat storage).

The idea is to use a Primary tank (1,000 litres) for heat storage for showers, etc (similar to no. 3 above), and a Secondary tank (50,000 litres) to collect the surplus heat generated in summer, and to use that heat for central heating in winter.  I’ve done the sums for this, and it looks feasable.  Here’s a diagram of how it might work:

Notes: The insulation around the tank would be 250mm thick polystyrene (wall panels from coolstore or chiller) which would retain heat fairly well (700 watt rate of heat loss, for 24 hrs per day = 0.7 x 24 = 17kWh heat lost per day.  Not too bad, since the solar panel heat gain is 50kWh per day in summer, and 20kWh per day in winter.  This should mean that the tank will heat up from 30deg.C to 70deg.C during the spring, summer and autumn, then it would cool back to 30deg.C when the hot water is used for central heating in the winter.

WHAT IS POWER?  WHAT IS ENERGY?

Some terms and definitions:

Just so that we all speak the same language, it's important to know the relationship between power and energy, and how we measure them.  Many people confuse the two things, when they are actually very different indeed.

Energy can always be converted into Work, and can come in many different forms, e.g.:

Chemical energy, which we buy from the petrol company, drives our car or mower

Electrical energy, which we buy from the national grid, drives our heater, jug, etc

Power on the other hand, is the rate of consumption of energy, and is a property of the machine consuming the energy, e.g.:

A car with a big engine is more powerful and uses chemical energy more quickly than a car with a small engine.  However, they can both do the same amount of work, but the small car will just take longer to do it.

An electric jug is more powerful and uses electrical energy more quickly than an electric light bulb.

Note:  We buy ENERGY, not POWER (despite the name “power company”)  Power is HOW FAST we use the Energy that we buy.  Try to remember this distinction, it's important.

In the example above, we buy chemical energy in Litres and electrical energy in  KiloWatt-hours.  A litre of chemical energy costs around $2, and a KiloWatt-hour of electrical energy costs around 25c.  A typical 2-person home relying solely on the grid for energy uses about 15KWh per day, averaged over the year.  This breaks down to 5KWh for hot water heating, 5KWh for space heating, and 5KWh for everything else.  (I call it the 5+5+5 rule).  This comes to around 6000 Kwh per year, costing $1500 per year.  In addition, we pay around $500 per year for the line charge.  A 4-person home would use nearly twice as much energy, i.e. around 10,000 Kwh per year, costing around $2500.

When we contemplate alternative hot water or electricity systems, we need to keep the above consumption and costs in mind.  Suppliers of solar and hydro systems should be able to give you figures for the energy which their products will provide to replace the energy you buy from the grid.

Right, now some maths:

Power is defined as:  the “Rate of consumption of Energy”,

(The higher the Power rating of the appliance, the faster it uses up energy)

We'll measure POWER in KiloWatts  (kW)

Energy consumed is the application of POWER for a certain TIME

Therefore, Energy = Power x Time - remember this formula!  We'll use it often.

We'll measure ENERGY in KiloWattHours (kWh).  i.e. kW x Hrs

(This is the same “unit” the electricity companies use when they charge you for the electrical energy you consume.)

Example 1:

A 2-bar electric heater has a Power rating of 2 kW.

If you use it for 3 hours in a day, you'll consume 2 x 3 = 6 kWh of energy.

Let's say you use the 2kW heater for 12 hours each day, because it's cold.

The 2-bar heater has a Power of 2 kW, used for a Time of 12 hours:

Energy = Power x Time    = 2 x 12    = 24 kWh

(At 25c per kWh, you would pay $6 per day, or $180 per month for this energy)

Example 2:

An electric jug (used to make four cups of coffee each day)

The Jug has a Power of 2.4 kW, used for 8 minutes each day: (that's 0.13 of an hour)

Energy = Power x Time   = 2.4 x 0.13  = 0.312 kWh

(At 25c per kWh, that's 8c per day, or $2.40 per month for this energy)

So, an electric heater left on for 12 hours consumes MUCH more energy per day than the jug!  (nearly 100 times as much, in fact – so you could make nearly 400 cups of coffee for the same energy cost!)  Although you’ll note that the jug is more POWERFUL than the heater.

Example 3:

How about electric lights?

A 100 Watt light bulb, left ON for a time of 24 hours – let's work out the Energy:

100 Watts is a Power of 0.100 kW  (1000 Watts = 1 kW, remember?)

Energy = Power x Time   = 0.100 x 24   =  2.4 kWh per day

(That's 60c per day,or $18 per month, or 7 times as much as 4 cups of coffee per day!)  But still only 1/10 of the cost of the heater.

Example 4:

A fridge or freezer is similar to a light-bulb, around 100 Watts.  But they cycle on and off with a 50% “duty cycle” approximately.  This means they are ON for only half the time on average.  So a fridge uses around 1.2 kWh per day = $0.30 ($9 per month)

Summary:

2-Bar electric heater for 12 hours:  	24kWh per day	$6.00 ($180 per month)

Electric Jug for 4 cups of coffee:  	0.312kWh per day	$0.08 ($2 per month)

100 Watt light bulb for 24 hours:		2.4kWh per day	$0.60 ($18 per month)

Fridge or freezer:				1.2kWh per day	$0.30 ($9 per month)

CALCULATING POWER AND ENERGY FOR A SOLAR HOT WATER SYSTEM

The Sun shines with an average power of 1 kW per square meter on the earth's surface.

This is for an average NZ Summer's day, between 10am and 3pm, i.e. for 5 hours.

(For Winter, you can work on 0.7 kW per square meter for 3 hours)

This means that if you can collect it with a 100% efficiency, each square meter can produce 1kW x 5 hrs  =  5kWh of energy per day in Summer, and 0.7kW x 3 hrs = 2.1kWh of energy per day in Winter.  (Remember, Power x Time = Energy)

A 4-person family requires about 10kWh of hot water heating per day.  This can be provided by a 2 sq.m. solar panel in Summer, or by a 5 sq.m. panel in Winter. (This assumes that the panel is 100% efficient! - which is impossible!)  This is why, when purchasing a solar hot water panel, you need to know how big it is, and how much energy it will produce per day – or you may find yourself disappointed with the performance...

Calculating the Power - “Single-Pass” method:

You need to know the Flow of water through the panel in Litres per Hour, and the Rise in Temperature from the inlet to the outlet end.  The Power is then calculated by this formula:  Power = Flow x TempRise / 857  (The answer comes out in kW)

VITAL STATISTICS OF A SOLAR HOT WATER SYSTEM

Definitions and Formulae:

Powersun = Sun's radiation on the surface of the earth at midday

Powersun = 1 kW per square metre of surface area exposed to the sun (summer)

Powersun = 0.7kW per square metre of surface area exposed to the sun (winter)

Poweract = Actual power absorbed into water, determined by measuring a rise in temperature of the water flowing at a certain number of litres per hour through the panel.

Poweract = Flow x Temprise / 857

Energy generated per day = Poweract x no. of hours of full sunlight

Energy generated per day = Poweract x 5 (summer)  or Poweract x 3 (winter)

Example:  Fill and Albie's “thermonuclear” hot water panel

| Area of panel in Square Metres | 30 Sq m |  |
| --- | --- | --- |
| Powersun (summer = area x 1) (winter = area x 0.7) | 30 kW (summer) 21 kW (winter) |  |
| Flow in Litres per Minute | 2.5 litres per min |  |
| Flow in Litres per Hour (L/min x 60) | 2.5 x 60 = 150 litres per hr |  |
| Water inlet temperature | 18 degrees |  |
| Water outlet temperature | 60 degrees |  |
| Temprise (outlet temp – inlet temp) | 60 – 18 = 42 degrees |  |
| Poweract (= Flow x Temprise / 857) | 150 x 42 / 857 = 7.4 kW |  |
| Efficiency (= Poweract / Powersun) | 7.4 / 30 = 0.25 (or 25%) |  |
| Energy generated per day in summer (= Poweract x 5 hrs) | 7.4 x 5 = 37 kWh (“units”) |  |

Some rules of thumb and formulae:

Sun’s Radiation:  	Summer:1kW per sq.m. (for 5 hrs per day)

Winter: 0.7kW per sq.m (for 3 hrs per day)

Efficiency of solar panels:  Polyethylene pipe under Agphane: 25%

Flat plate (steel or copper) under glass: 50%

Evacuated glass tubes: 70%

Panel Power = Panel area x Sun’s radiation x panel Efficiency

Energy produced per day = Panel Power x hrs of sunlight

E.G. a 24 sq. m. panel (polyethylene) in winter:

Panel area = 24,  Sun’s Radiation = 0.7,  Efficiency = 0.25, hrs of sunlight = 3

Thus: 	Energy produced per day 	= 24 x 0.7 x 0.25 x 3

= 12.6 kWh per day.

Energy required to heat water:  (using a “proportion” rule)

1 kWH will heat 100 Litres through 8.6 degrees C  (Always start with this one…)

2 kWH will heat 200 Litres through 8.6 degrees C

5 kWH will heat 500 Litres through 8.6 deg.C

10 kWh will heat 1000 Litres through 8.6 deg. C

etc…

OR:

1 kWH will heat 100 Litres through 8.6 deg.C

2 kWH will heat 100 Litres through 17.2 deg.C

5 kWH will heat 100 Litres through 43 deg.C

(This one only works below boiling point!)

E.G. What size panel do I need to heat a 1000 Litre tank from 40 to 60 deg. each day?

1 kWH will heat 100 Litres through 8.6 deg.C

2.3 kWH will heat 100 Litres through 20 deg.C  (I’ve multiplied by 2.3)

23 kWH will heat 1000 Litres through 20 deg.C  (I’ve multiplied by 10)

This means I’ll need a panel about twice as large as the one in the example above, to get 23 kWH per day.  (But then, I’ll have enough hot water in winter to heat my cottage!!)

I hope these notes are useful, and that you enjoyed the seminar!

Albie Burgers, August 2011.