# Fill and Albie’s “Li’l Blue” All-Electric Daihatsu

# (For more info please Google for “Fill and Albie Electric Car”)

Purchase Date:	10 October 2013

Make and Model:	Daihatsu Charade (1988  Petrol)

Converted to electric in August 2008:

Old Batteries:	6 x 12V 130AH Lead-Acid deep cycle, in series.

These batteries failed fairly quickly, so when we bought the car

we replaced them with Lithium Ferrous Phosphate cells

New Batteries: 	48 x 3.2V 100AH  LiFePO4, (24 parallel pairs, in series)

Can draw 300A continuous, 1000A peak

Should last for 5000 – 7000 cycles (about 25 years)

Energy Stored:	200AH at 80V  (16-20kWH), giving a Range of 100km

Motor:	72V Mars Electric Brushed Permanent Magnet motor

200A continuous current, 500A peak (for 1 minute)

Power: 11.5kW continuous, 22.3kW peak (for 1 minute)

## Controller:	Kelly KDZ72551, 220A continuous, 550A peak

Range:	100km  (comfortably to the Mussel Inn and back – Twice!)

- We charge the batteries at 10-20ADC (they can accept charge at 100A, but our current charger can’t do this.)  “current” – haha.

- The batteries take about 7 hours to charge fully from nearly empty. (Using our 20A charger at 80VDC, about 2kW)

- We charge from our own solar panels, so the car is truly “Solar Powered”!

- When we go downhill, the motor acts as a generator, and regains about 50% of the energy used to get up the hill.

- When we bought the car, its lead-acid batteries were almost dead, so we changed to Lithium batteries. This cost $8,000, 3 times as much as lead acid, but because they should last 25 years, they make economic sense!

- Our speed is limited by the Kelly Controller, which only provides 220A continuous current, and the motor (200A continuous)  This allows us to travel at 60 – 80 km/hr on the flat.  Hills are a bit more of a challenge.  We hope people behind us will be understanding.

- The car costs 1.5c/km to run if we charge up on our solar panels.  If we purchase electricity at retail rates, then it costs 3.5c/km.  This translates to a petrol equivalent of 0.68 litres/100km or 1.6 litres/100km respectively.

A good reason for investing in Li’l Blue is to show that we can wean ourselves off fossil fuels for car transport.  We don’t have to burn millions of litres of non-replaceable petrol to carry 3-ton cages of steel around with us.  We can build super-light cars with batteries weighing less than 100kg, and charge them up on renewable energy.  Imagine e-car charging points scattered around our districts, perhaps with associated coffee shops, where we can gather with like-minded folk and solve the problems of the world while our cars fill up with electrons.  Better still, how about replacing cars altogether, and getting onto an electric bike?  Now there’s a thought….

(Felicity and Albie Burgers – October 2015)