A Grid-connected Solar PV Home - an Investigation.  (Albie Burgers November 2011)

This is a thought experiment, to see if Solar PV panels are a viable option for a household, in order to offset some of the electricity produced by coal and gas in New Zealand.  Our home uses about 20kWh per day, so the system is sized accordingly.

Some assumptions:

- The house is already connected to the grid, so this will be a “grid-connect” system where surplus electricity generated goes directly into the national grid.  When the house uses more electricity than the panels can generate at that time, the flow of electricity is reversed.  The system thus uses the national grid as a “battery”, and no batteries are installed.

- The system under investigation produces a maximum power output of 5kW, and generates 20kWh of energy per day, averaged over a year.  (That's an average of 3 hours per day of bright sunshine in winter, and 5 hours in summer)

- The surplus energy is sold to the grid at “par”, which means at the same value per unit (kWh) as energy purchased from the grid.  This value is assumed to be 25c per unit, including GST, increasing at 2% per year.

- The total installed cost of the system is $25,000, including equipment, consents, meeting all the requirements of the network company for equipment specifications, installation and connection to the grid.

- The solar panels have a lifespan of 40 years, during which they produce 100% of their rated output for the first 10 years, 90% for the next 10 years, 80% for the next 10 years, and 70% for the last 10 years.  At the end of 40 years, it is assumed that technology will have moved on, and other forms of generation will be available, so the value of the system will have reduced to zero.

Embedded Energy

This is the energy consumed in the manufacture of the equipment, and which needs to be taken into account because we want to be sure that the equipment will generate substantially more energy in its lifetime than was consumed in its manufacture.  (Otherwise, there is no point to the exercise...)

A number of studies have been made on this topic, and the number of years of production required to equal the embedded energy has been estimated by most studies as being between 3 and 5 years.   A few studies estimated between 10 and 15 years, and only one (which has  been widely quoted by detractors) made a claim of “longer than the lifetime” of the panels. (Ref:  “Energy Payback of Roof Mounted Photovoltaic Cells” by Colin Bankier and Steve Gale, Published Jun 16 2006 by Energy Bulletin)

Thus it is reasonable to assume that the embedded energy is recovered early on in the lifetime of the project.

Revenue Recovery (return on investment)  - See separate spreadsheet for calculations.

It is important that the system provides a reasonable return on the substantial investment over the 40-year life of the project.  It should not only pay for the capital cost, but should provide “interest” as though the money had been invested in a bank.

Using the figures above, the total cost of the system ($25,000) is paid back in 13 years.

The next 27 years of income can be considered as the “interest” on the investment.

This comes to a total of $66,000, which represents interest on $25,000 for 40 years.  The interest rate per annum works out at 6.6%.  If the installation is part of a business venture (e.g. a farm or a trust), then GST can be claimed on the purchase, and the capital cost can be depreciated over, say, 25 years.  If the IRD choose to consider personal income from such systems to be taxable, then a business venture may be the way to go.

# Offsetting Fossil Fuel Generation

New Zealand generates about 26% of its electricity by burning coal, gas and oil. (Ref: Wikipedia)  Most of the rest is hydro, which is fortunate, because hydro lakes act as a storage “battery” which is recharged when alternative energy sources such as PV, wind and tide come on stream.  For example, when the sun shines and PV systems start generating, hydro generation decreases automatically, and the dams start to fill up and store the PV energy.  This evens out the uneven generation of these PV sources, which would otherwise be a real problem!  In addition, as the hydro lakes fill up and reach capacity, coal and gas generators are taken off-line. This means that virtually every unit of electricity produced by PV represents a unit NOT produced by coal or gas.

# Line Losses and Distributed Generation

Line losses on the grid amount to around 12% as the electricity moves from areas of surplus to areas of demand.  (Most of this energy is lost as heat, because the lines are made of aluminium and have a fairly high resistance.)  If we can distribute the generation around the grid, these line losses are minimised.  Effectively this means that every 100 units of energy generated by PV and used locally, saves the lines company 12 units, which can be sold to someone else!  It also means that building new generating plant can be deferred.  The capital cost of PV generation equipment is also borne by individual householders, not the generating companies, which means that ownership of the generation is also distributed, as well as the generation itself.

Conclusions:

- The cost of this type of generation is still fairly high, but as the figures above show, the return on investment is reasonable. We could wait until PV equipment prices fall further (as they surely will), or until our government introduces more incentives for PV generation (which is less certain), but we think the investment is sound already and will only get better as electricity prices rise.

- In New Zealand, we are well placed to take advantage of intermittent renewable generation like PV, because of the storage capability of our hydro reservoirs.  PV generation is better than carbon neutral because of the rapid repayment of the embedded energy cost, and the subsequent offsetting of fossil fuel energy generation.

- We need local, enthusiastic and knowledgeable plumbers, electricians, builders and accountants with the skills and will to help people set up systems like this, all over New Zealand.

- We would like to use our system as an example of how an ordinary family with a modest amount of money to invest can achieve something positive with a little help from local enthusiastic experts.

- We would love to see every North-facing rooftop in New Zealand covered with solar PV or hot water panels.  Wouldn’t it be great?

Albie and Felicity Burgers, November 2011